# DC Motor Speed Control with Obstacle Detection

## Objective

Control a DC motor speed using PWM and automatically stop the motor when an obstacle is detected within a distance range of 3 cm to 325 cm using an ultrasonic sensor.

# DC Motor Speed Control with Obstacle Detection

## Objective

Control a DC motor speed using PWM and automatically stop the motor when an obstacle is detected within a distance range of 3 cm to 325 cm using an ultrasonic sensor.

## Components

- Arduino UNO (or ESP32)

- L293D Motor Driver

- DC Motor

- HC-SR04 Ultrasonic Sensor

- Breadboard and jumper wires

- External motor power supply (if needed)

## Wiring Diagram

+5V

|

VCC

HC-SR04

Trig -> D8

Echo -> D9

GND -> GND

DC Motor -> L293D OUT1 & OUT2

IN1 -> D2

IN2 -> D3

EN1 -> D5 (PWM)

Vcc2 -> Motor power supply (5-12V)

GND -> GND

Notes:

- EN1 pin controls motor speed via PWM.

- IN1 and IN2 control motor direction.

- Trig sends ultrasonic pulses; Echo receives reflected pulses to measure distance.

## Code Functionality

- Motor runs forward at a set PWM speed (motorSpeed).

- HC-SR04 continuously measures distance.

- Motor stops if distance ≥ 3 cm and ≤ 325 cm.

- Motor runs normally if no obstacle is detected outside this range.

## Test Results (Example)

| Distance (cm) | Motor Status |
| --- | --- |
| 50 | Running |
| 10 | Stopped |
| 330 | Running |
| 3 | Stopped |

Observation: Motor responds quickly to obstacles within the defined range, maintaining safety and preventing collisions.

💻 Arduino Code

// Motor pins

const int pwmPin = 5;  // PWM output (hardware PWM)

const int dir1   = 4;

const int dir2   = 7;

// Ultrasonic pins

const int trigPin = 10;

const int echoPin = 9;

// Stop range

const int MIN_STOP = 3;    // cm

const int MAX_STOP = 325;  // cm

void setup() {

pinMode(pwmPin, OUTPUT);

pinMode(dir1, OUTPUT);

pinMode(dir2, OUTPUT);

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

Serial.begin(9600);

// Motor initial direction

digitalWrite(dir1, HIGH);

digitalWrite(dir2, LOW);

}

long readDistance() {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH);

long distance = duration * 0.034 / 2; // cm

return distance;

}

void loop() {

long dist = readDistance();

Serial.print("Distance: ");

Serial.print(dist);

Serial.println(" cm");

// Stop motor if distance is in the 3–325 cm range

if (dist >= MIN_STOP && dist <= MAX_STOP) {

analogWrite(pwmPin, 0); // stop

Serial.println("Motor stopped");

} else {

analogWrite(pwmPin, 200); // run motor at fixed speed outside range

Serial.println("Motor running");

}

delay(100); // 100ms loop

}